Chapter 11: Sleep Diagnostics

MULTIPLE CHOICE

1.	Which of the following brain waveforms are present during wakefulness?

1. Alpha waves

2. Beta waves

3. Relatively low-voltage, mixed-frequency waves

4. Delta waves

| a. | 1 and 2 | c. | 3 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1, 2, and 4 |

ANS:	A

An electroencephalogram shows that alpha waves and beta waves are present during wakefulness and diminish as the sleeper’s electroencephalogram converts to relatively low-voltage, mixed-frequency waves. Delta waves are present during slow-wave sleep or the deepest stages of non-REM sleep.

PTS:	1	REF:	Page 346

2.	Stage 2 non-REM sleep is characterized by the presence of which of the following?

1. Relatively low-voltage, mixed-frequency waves

2. Sleep spindles

3. Delta waves

4. K-complexes

| a. | 1 and 3 | c. | 1 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 4 | d. | 2, 3, and 4 |

ANS:	D

Stage 2 non-REM sleep is identified on a sleeper’s electroencephalogram by the appearance of sleep spindles and K-complexes. Relatively low-voltage, mixed-frequency waves occur during non-REM stage 1 sleep. Delta waves are present during slow-wave sleep or non-REM sleep stages 3 and 4.

PTS:	1	REF:	Page 346

3.	Which characteristics are associated with REM sleep?

1. Lower level tonic activity

2. High-voltage, low-frequency waves on an electroencephalogram

3. Absence of tonic activity on an electromyogram

4. Often absent responsiveness to external stimuli

| a. | 1 and 2 | c. | 1, 2, and 4 |
| --- | --- | --- | --- |
| b. | 3 and 4 | d. | 2, 3, and 4 |

ANS:	B

Absence of tonic activity on an electromyogram and an often absent responsiveness to external stimuli are associated with REM sleep. Lower level tonic activity on an electromyogram and high-voltage, low-frequency waves on an electroencephalogram are indicative of non-REM sleep.

PTS:	1	REF:	Page 346, Table 11-1

4.	Sleep spindles occur at a frequency of ____ cycles/s.

| a. | 4 to 7 | c. | 14 to 16 |
| --- | --- | --- | --- |
| b. | 9 to 13 | d. | <3.5 |

ANS:	B

Sleep spindles are waveforms with waxing and waning amplitude that occur at a frequency of 9 to 13 cycles/s.

PTS:	1	REF:	Page 346

5.	The age at which infants first exhibit the classic sleep stages seen in adults is ____ months.

| a. | 6 | c. | 12 |
| --- | --- | --- | --- |
| b. | 9 | d. | 18 |

ANS:	C

By 12 months of age, infants exhibit the classic sleep stages seen in adults.

PTS:	1	REF:	Page 348

6.	During a typical night of sleep, a normal adult sleeper cycles between non-REM sleep and REM sleep at approximately every ____ minutes.

| a. | 30 to 60 | c. | 90 to 120 |
| --- | --- | --- | --- |
| b. | 60 to 90 | d. | 120 to 150 |

ANS:	C

During a typical night of sleep, a normal adult sleeper cycles between non-REM sleep and REM sleep approximately every 90 to 120 minutes.

PTS:	1	REF:	Page 348

7.	Normally, the number of cycles of sleep stages that occur per night is ____ cycles.

| a. | 4 to 6 | c. | 8 to 10 |
| --- | --- | --- | --- |
| b. | 6 to 8 | d. | 10 to 12 |

ANS:	A

Normally, 4 to 6 cycles of sleep stages occur per night.

PTS:	1	REF:	Page 348

8.	Which stage(s) of sleep is (are) shown in the figure?

| a. | Stage 1 sleep | c. | Stages 3 and 4 sleep |
| --- | --- | --- | --- |
| b. | Stage 2 sleep | d. | Rem sleep |

ANS:	B

Stage 2 sleep is characterized by relatively low-voltage background electroencephalographic activity, sleep spindles and K-complexes, absence of eye movements, and tonic activity on an electromyogram.

PTS:	1	REF:	Page 347, Figure 11-1B

9.	Which stage(s) of sleep is (are) shown in the figure?

| a. | Stage 1 sleep | c. | Stages 3 and 4 sleep |
| --- | --- | --- | --- |
| b. | Stage 2 sleep | d. | Rem sleep |

ANS:	D

REM sleep is characterized by relatively low-voltage, mixed-frequency background electroencephalographic activity, with a burst of notched theta waves; rapid, saccadic, conjugate eye movements; chin muscle tone significantly decreased from that found with waking; and non-REM sleep levels.

PTS:	1	REF:	Page 347, Figure 11-1D

10.	Which sleep stage(s) is (are) characterized normally by irregular breathing patterns with short, central periods of apnea and inhibited rib cage muscles and upper airway muscles?

| a. | Stage 1 sleep | c. | Stages 3 and 4 sleep |
| --- | --- | --- | --- |
| b. | Stage 2 sleep | d. | Rem sleep |

ANS:	D

REM sleep is characterized by the following physiologic effects on respiration: irregular pattern of breathing; short, central apneas; variable and elevated PaCO2 (2-8 mm Hg above wakefulness); inhibited rib cage muscles; inhibited upper airway muscles; decreased chemoresponsiveness; and high threshold of arousability to respiratory stimuli.

PTS:	1	REF:	Page 348, Table 11-2

11.	Which statements are true concerning non-REM sleep?

1. Early stages predispose a person to apneic periods.

2. PaO2 levels decrease by 5 to 10 mm Hg.

3. Minute ventilation increases by 1 to 2 L/min during stages 3 and 4.

4. The amount of time spent in these stages is less than 50% of the total sleep time.

| a. | 1 and 2 | c. | 1 and 4 |
| --- | --- | --- | --- |
| b. | 3 and 4 | d. | 2 and 4 |

ANS:	A

The reduction in respiratory drive that occurs during the early stages of non-REM sleep (stages 1 and 2) predisposes the person to apneic periods during fluctuations between being awake and asleep. With the establishment of non-REM slow-wave sleep (stages 3 and 4), nonrespiratory inputs are minimized, and minute ventilation is regulated by metabolic control. Minute ventilation decreases by 1 to 2 L/min compared with wakefulness. As a consequence, PaCO2 increases by 2 to 8 mm Hg, and PaO2 decreases by 5 to 10 mm Hg.

PTS:	1	REF:	Pages 348-349

12.	Which of the following constitute the effects of normal sleep on the cardiovascular system?

1. Tachycardia during slow-wave sleep

2. Vasoconstriction during slow-wave sleep

3. Hypotension with significant phasic elevations during REM sleep

4. Slightly decreased cardiac output during non-REM and REM sleep

| a. | 1 and 2 | c. | 1 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 3 and 4 |

ANS:	D

During REM sleep, there is hypotension with significant phasic elevations in blood pressure. During non-REM and REM sleep, cardiac output is slightly decreased. Bradycardia and vasodilation occur during slow-wave sleep.

PTS:	1	REF:	Page 348, Figure 11-3

13.	Standard polysomnography includes which of the following?

1. Esophageal pH

2. Submental electromyogram

3. Electrooculogram

4. Intrapleural pressure

| a. | 1 and 2 | c. | 2, 3, and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1, 2, 3, and 4 |

ANS:	B

Standard polysomnography includes electroencephalogram, electrooculogram, submental electromyogram, tibialis electromyogram, breathing pattern, SpO2, PCO2, and electrocardiogram. Intrapleural pressure and esophageal pH are special procedures that may be required after polysomnography.

PTS:	1	REF:	Page 350, Table 11-4

14.	Monitoring of respiratory effort during polysomnography may be accomplished by using which of the following?

1. Impedance pneumography

2. Pneumotachograph at airway opening

3. Strain gauge belt around the abdomen

4. Respiratory inductance plethysmography

| a. | 1 and 2 | c. | 2, 3, and 4 |
| --- | --- | --- | --- |
| b. | 3 and 4 | d. | 1, 2, 3, and 4 |

ANS:	D

All of the above-listed equipment can be used to assess and measure respiratory effort during polysomnography.

PTS:	1	REF:	Page 350

15.	The submental electromyogram electrodes are placed in which of the following positions?

1. One on the center of the chin

2. Two along the jawline, 3 cm apart

3. Two on the hyoid bone lateral to each other and 2 cm apart

4. One on either side of the chin, between the chin and hyoid bone, 2 cm apart

| a. | 1 and 2 | c. | 2 and 3 |
| --- | --- | --- | --- |
| b. | 1 and 4 | d. | 3 and 4 |

ANS:	B

Three electrodes are typically affixed to the chin with tape. Two of these electrodes are placed between the tip of the chin and the hyoid bone, lateral to each other and 2 cm apart; a third electrode is placed in the center of the chin.

PTS:	1	REF:	Page 352

16.	REM sleep is identified by which of the following measurements?

| a. | Electrocardiogram | c. | Electromyogram |
| --- | --- | --- | --- |
| b. | Electroencephalogram | d. | Electrooculogram |

ANS:	D

The electrooculogram records eye movements during sleep and allows the clinician to differentiate non-REM sleep states from REM sleep states. An electromyogram records arousal responses and sleep movements. An electroencephalogram measures brain wave activity during sleep. An electrocardiogram measures cardiac activity.

PTS:	1	REF:	Page 353

17.	The electrooculogram electrodes should be placed in which of the following positions?

| a. |  | c. |  |
| --- | --- | --- | --- |
| b. |  | d. |  |

ANS:	B

Electrodes are placed on the skin surface in the periorbital region, specifically approximately 1 cm lateral to the outer canthi of the eyes and offset from the horizontal plane (i.e., 1 cm above the horizontal plane on one side and 1 cm below the horizontal plane on the other side).

PTS:	1	REF:	Page 252

18.	Electrooculogram electrodes are in place, and eye movement is to the left. The electrooculogram recording is shown by which of the following?

| a. |  | c. |  |
| --- | --- | --- | --- |
| b. |  | d. |  |

ANS:	B

The height or depth of the deflection depends on the movement of the eyes relative to the fixed electrodes. Eye movement away from the electrode shows an upward deflection on the electrooculogram, and eye movement toward the electrode shows a downward deflection.

PTS:	1	REF:	Page 353, Figure 11-7

19.	Electrooculogram electrodes are in place, and the eyes are looking up. The electrooculogram recording is shown by which of the following?

| a. |  | c. |  |
| --- | --- | --- | --- |
| b. |  | d. |  |

ANS:	D

The height or depth of the deflection depends on the movement of the eyes relative to the fixed electrodes. Eye movement away from the electrode shows an upward deflection on the electrooculogram, and eye movement toward the electrode shows a downward deflection. When looking up with the electrodes in place, the left eye has moved away from its electrode, and the right eye has moved toward its electrode. Because the movement is not directly toward either right or left, the deflections produced by either electrode are smaller than left or right movement.

PTS:	1	REF:	Page 353, Figure 11-7

20.	Which waveforms are present during wakefulness and during the onset of quiet sleep?

1. Alpha waves

2. Beta waves

3. Delta waves

4. Theta waves

| a. | 1 and 2 | c. | 3 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1 and 4 |

ANS:	A

Alpha and beta waves are present during wakefulness and during the onset of quiet sleep.

PTS:	1	REF:	Page 346

21.	Slow, rolling, pendulous movement of the eyes is present during which of the following sleep stages?

| a. | Stage 1 sleep | c. | Stages 3 and 4 sleep |
| --- | --- | --- | --- |
| b. | Stage 2 sleep | d. | Rem sleep |

ANS:	A

This type of eye movement is indicative of stage 1 sleep.

PTS:	1	REF:	Page 346

22.	What is the distinguishing feature of the electroencephalogram during sleep stages 3 and 4?

| a. | Small beta waves | c. | Sleep spindles |
| --- | --- | --- | --- |
| b. | Large k-complexes | d. | Large delta waves |

ANS:	D

The deepest stages of non-REM sleep (stages 3 and 4) are referred to as slow-wave sleep because of the presence of large delta waves that appear when the sleeper enters these stages.

PTS:	1	REF:	Page 346

23.	Sleep stage 3 is distinguished from sleep stage 4 by the:

| a. | Presence of bursts of theta waves |
| --- | --- |
| b. | Presence of sleep spindles and k-complexes |
| c. | Percentage of large delta waves present on the electroencephalogram |
| d. | Percentage of relatively low-voltage, mixed-frequency waves |

ANS:	C

The distinction between stage 3 and stage 4 is usually found in the presence of large delta waves, or slow waves. At the beginning of stage 3, delta waves constitute 20% to 50% of the electroencephalogram. Stage 4 is identified by the presence of the delta waves for at least 50% of the electroencephalogram recording.

PTS:	1	REF:	Page 346

24.	A sleeper is predisposed to periods of apnea during which of the following non-REM sleep stages?

1. Stage 1

2. Stage 2

3. Stage 3

4. Stage 4

| a. | 1 and 2 | c. | 3 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1 and 4 |

ANS:	A

Non-REM stages 1 and 2 predispose individuals to apneic periods because of the reduction in respiratory drive and the fluctuations between being awake and asleep. Apnea is likely again during REM sleep because of the reduction in response to chemical and mechanical respiratory stimuli.

PTS:	1	REF:	Page 349, Table 11-3

25.	What are the chief symptoms associated with obstructive sleep apnea?

1. Nosebleeds

2. Insomnia

3. Morning headaches

4. Daytime sleepiness

| a. | 1 and 4 | c. | 2 and 3 |
| --- | --- | --- | --- |
| b. | 2 and 4 | d. | 3 and 4 |

ANS:	D

The symptoms most commonly associated with obstructive sleep apnea in adult patients are chronic loud snoring, gasping or choking episodes during sleep, excessive daytime sleepiness, morning headaches, and personality and cognitive deterioration related to fatigue from lack of sleep.

PTS:	1	REF:	Page 354

26.	The criteria for diagnosing obstructive sleep apnea with polysomnography include which of the following?

| a. | More than 2 periods of obstructive apnea, of 15 seconds’ duration each, per hour |
| --- | --- |
| b. | More than 5 periods of obstructive apnea, of 10 seconds’ duration each, per hour |
| c. | At least 5 periods of obstructive apnea, of 30 seconds’ duration each, per hour |
| d. | At least 10 periods of obstructive apnea, of 20 seconds’ duration each, per hour |

ANS:	B

Part of a diagnosis of obstructive sleep apnea requires that the patient demonstrate more than 5 periods of obstructive apnea, each of 10 seconds’ duration, per hour of sleep.

PTS:	1	REF:	Page 355, Box 11-2

27.	A patient diagnosed with obstructive sleep apnea would have which of the following polysomnographic testing results?

| a. | 8 apnea periods, of 4 to 8 seconds’ duration, per hour of sleep and an SpO2 of 90% to 93% |
| --- | --- |
| b. | 20 apnea periods, of 5 to 9 seconds’ duration, per hour of sleep, with no arousals |
| c. | 15 apnea periods, of 15 to 20 seconds’ duration, per hour of sleep and bradycardia |
| d. | 5 apnea periods, of approximately 8 seconds’ duration each, per hour of sleep, with snoring |

ANS:	C

This patient has more than 5 apnea periods per hour of sleep with each episode lasting more than 10 seconds, along with bradycardia.

PTS:	1	REF:	Page 355, Box 11-2

28.	Patients who are at highest risk for developing obstructive sleep apnea include patients with which of the following?

1. Asthma

2. Nuchal obesity

3. Neuromuscular disorders

4. Nasopharyngeal narrowing

| a. | 1 and 3 | c. | 2 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1 and 4 |

ANS:	C

Patients at the greatest risk of developing obstructive sleep apnea are patients with obesity, especially nuchal obesity, and patients with nasopharyngeal narrowing.

PTS:	1	REF:	Pages 354-355

29.	Which of the following factors increase a patient’s risk for obstructive sleep apnea?

1. Systemic hypertension

2. Exercising late in the day

3. Excessive coffee drinking

4. Alcohol consumption close to bedtime

| a. | 1 and 2 | c. | 3 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1 and 4 |

ANS:	D

Some factors that add to the risk of obstructive sleep apnea include systemic hypertension, ingestion of central nervous system depressants, and alcohol consumption close to bedtime.

PTS:	1	REF:	Pages 354-355

30.	Which factors may augment the symptoms of patients with mild obstructive sleep apnea?

1. Excessive day-shift work

2. Sleep deprivation

3. Respiratory allergies

4. Scuba diving

| a. | 1 and 2 | c. | 3 and 4 |
| --- | --- | --- | --- |
| b. | 2 and 3 | d. | 1 and 4 |

ANS:	B

Factors that may increase the symptoms of patients with mild obstructive sleep apnea include partial sleep deprivation, as with shift work; respiratory allergies; exposure to smoke; and ascent to altitude.

PTS:	1	REF:	Page 354

31.	Which oxygen desaturation level is generally considered abnormal in children?

| a. | <98% | c. | <92% |
| --- | --- | --- | --- |
| b. | <95% | d. | <90% |

ANS:	C

Oxygen desaturation of less than 92% is generally considered abnormal in children, depending on their baseline oxygen saturation.

PTS:	1	REF:	Page 356

32.	Which criteria related to measurements of PETCO2 are used to determine sleep-disordered breathing in children?

| a. | PETCO2 values of >8 mm Hg above baseline |
| --- | --- |
| b. | PETCO2 values of >12 mm Hg above baseline |
| c. | PETCO2 values of >40 mm Hg for at least 25% of total sleep time |
| d. | PETCO2 values of >45 mm Hg for at least 60% of total sleep time |

ANS:	D

For children, it has been suggested that PETCO2 values of greater than 45 mm Hg for at least 60% of the total sleep time or PETCO2 values greater than 13 mm Hg above baseline values indicate sleep-disordered breathing.

PTS:	1	REF:	Page 356

33.	Which of the following is a common problem among patients with central sleep apnea?

| a. | Obesity | c. | Depression |
| --- | --- | --- | --- |
| b. | Insomnia | d. | Hypersomnolence |

ANS:	C

Depression is a common finding in patients with central sleep apnea. Patients with central sleep apnea do not normally report insomnia and hypersomnolence and typically have a normal body habitus.

PTS:	1	REF:	Page 357

34.	Which of the following represents the polysomnographic tracing of a patient with central sleep apnea?

| a. |  |
| --- | --- |
| b. |  |
| c. |  |
| d. |  |

ANS:	D

The polysomnographic tracing of a patient with central sleep apnea shows a complete cessation of nasal and oral airflow during the apneic episode.

PTS:	1	REF:	Page 357, Figure 11-11

35.	Which of the following represents the polysomnographic tracing of a patient with obstructive sleep apnea?

| a. |  |
| --- | --- |
| b. |  |
| c. |  |
| d. |  |

ANS:	C

The polysomnographic tracing of a patient exhibiting obstructive sleep apnea shows paradoxical movement of the chest and abdomen and recurrent complete airway obstructions, leading to oxygen desaturation.

PTS:	1	REF:	Page 358, Figure 11-12

36.	The polysomnographic tracing of a patient with mixed apnea is shown by which of the following?

| a. |  |
| --- | --- |
| b. |  |
| c. |  |
| d. |  |

ANS:	A

With mixed apnea, there is cessation of airflow at the nose and mouth. Initially, there is an absence of respiratory effort, which is the central component, followed by at least 2 cycles of respiratory effort with continued absence of airflow, which is the obstructive component.

PTS:	1	REF:	Page 358, Figure 11-12

37.	A K-complex is which of the following?

| a. | A | c. | C |
| --- | --- | --- | --- |
| b. | B | d. | D |

ANS:	A

K-complexes are large, vertical, slow waves that have an amplitude of at least 75 V, with an initial negative deflection.

PTS:	1	REF:	Page 346

38.	What is the key to recognizing obstructive sleep apnea when the results of a sleep study are analyzed?

| a. | No airflow and no respiratory effort |
| --- | --- |
| b. | No airflow with increasing respiratory effort |
| c. | Reduced airflow with minimal respiratory effort |
| d. | No airflow with no or minimal respiratory effort |

ANS:	B

The finding of no airflow with increasing respiratory effort is needed to diagnose obstructive sleep apnea.

PTS:	1	REF:	Page 356, Figure 11-15

39.	The number of apneic periods observed is 15, and the number of hours slept is 5. The apnea index is:

| a. | 3 | c. | 20 |
| --- | --- | --- | --- |
| b. | 8 | d. | 45 |

ANS:	A

To determine the apnea index, divide the calculated periods by the number of episodes per hour.

PTS:	1	REF:	Page 353

40.	Which of the following is (are) potential applications of electromyography?

1. Monitoring activation of the muscles of the upper airway

2. Monitoring contractile strength of the heart

3. Monitoring activity of intercostal muscles

4. Monitoring peripheral perfusion

| a. | 1 only | c. | 1 and 3 |
| --- | --- | --- | --- |
| b. | 1 and 2 | d. | 2, 3, and 4 |

ANS:	C

The electromyogram can be used to monitor activation of the muscles that expand the upper airway and to monitor the intercostal muscles as an alternative to assess respiratory activity.

PTS:	1	REF:	Page 356

41.	When monitoring nasal airflow, what is the most common problem associated with the use of the pneumotachograph?

| a. | Change in temperature of expired gas |
| --- | --- |
| b. | Variability in expiratory flow rates |
| c. | Requirement to reposition the probe |
| d. | Recalibration of the sensor |

ANS:	C

The most common problem with the pneumotachograph relates to probe position.

PTS:	1	REF:	Page 352

42.	During polysomnography, the patient has 15 periods of apnea and 10 episodes of hypopnea during 6 hours of sleep.    What is their apnea hypopnea index?

| a. | 2.5 | c. | 4.2 |
| --- | --- | --- | --- |
| b. | 3.5 | d. | 5.6 |

ANS:	C

The apnea hypopnea index is calculated by taking the sum of the total number of apneas and the number of episodes of hypopnea. The total is then divided by the number of hours of sleep.

PTS:	1	REF:	Page 353